PACKET CAPTURE ANALYSIS

Zyxel GS1915-8EP  —  Nebula Connectivity Issue

For Zyxel Technical Support  |  Capture file: 26.pcapng

| Report Date | 14 April 2026 |
| --- | --- |
| Device Model | Zyxel GS1915-8EP |
| Firmware Version | V4.70(ACAQ.6) ¦ Build date: 12/19/2025 |
| Device MACs | f4:4d:5c:98:63:17 (management) / f4:4d:5c:98:63:19 (bridge/port) |
| Configured IP | 192.168.1.1 (observed via ARP sender field and LLDP management TLV) |
| Capture File | 26.pcapng |
| Capture Duration | 264.3 seconds |
| Total Frames | 5,022 |
| Capture Tool | Wireshark / tshark 4.2.2 |

# 1.  Devices Identified in Capture

The following devices were identified from MAC address OUI resolution, ARP sender/target fields, DHCP option 12 (hostname), and LLDP TLVs present in the capture.

| Device | MAC Address | IP Address | Identification Source |
| --- | --- | --- | --- |
| Zyxel GS1915-8EP | f4:4d:5c:98:63:17 | 192.168.1.1 | ARP sender IP (frames #2134, #3233); LLDP management address TLV |
| Zyxel GS1915-8EP | f4:4d:5c:98:63:19 | — | LLDP chassis ID; STP bridge identifier |
| Sagemcom Router | 08:7b:12:e5:ef:07 | 192.168.1.254 | ARP sender IP; DHCP server identifier option; STP root bridge ID |
| HP Laptop | e0:70:ea:52:a7:8a | 192.168.1.2 | DHCP option 12 hostname 'Fieldlaptop2'; ARP sender IP; DHCP ACK |

# 2.  Chronological Event Log  —  Zyxel Switch Traffic Only

The following table lists every frame transmitted by the Zyxel switch (source MAC f4:4d:5c:98:63:17 or f4:4d:5c:98:63:19) in chronological order. Frames from other devices are omitted from this table and are covered in Section 3.

| Frame | Time (s) | Source MAC | Proto | Detail | Note |
| --- | --- | --- | --- | --- | --- |
| #196 | 1.744 | f4:4d:5c:98:63:19 | STP | RST BPDU Root=32768/08:7b:12:e5:ef:07 Cost=4 Port=0x8002 | BPDU 1 of 18 |
| #356 | 3.745 | f4:4d:5c:98:63:19 | STP | RST BPDU Root=32768/08:7b:12:e5:ef:07 Cost=4 Port=0x8002 | BPDU 2 of 18 |
| #497 | 5.745 | f4:4d:5c:98:63:19 | STP | RST BPDU Root=32768/08:7b:12:e5:ef:07 Cost=4 Port=0x8002 | BPDU 3 of 18 |
| #644 | 7.745 | f4:4d:5c:98:63:19 | STP | RST BPDU Root=32768/08:7b:12:e5:ef:07 Cost=4 Port=0x8002 | BPDU 4 of 18 |
| #772 | 9.744 | f4:4d:5c:98:63:19 | STP | RST BPDU Root=32768/08:7b:12:e5:ef:07 Cost=4 Port=0x8002 | BPDU 5 of 18 |
| #961 | 11.746 | f4:4d:5c:98:63:19 | STP | RST BPDU Root=32768/08:7b:12:e5:ef:07 Cost=4 Port=0x8002 | BPDU 6 of 18 |
| #1066 | 13.746 | f4:4d:5c:98:63:19 | STP | RST BPDU Root=32768/08:7b:12:e5:ef:07 Cost=4 Port=0x8002 | BPDU 7 of 18 |
| #1215 | 15.746 | f4:4d:5c:98:63:19 | STP | RST BPDU Root=32768/08:7b:12:e5:ef:07 Cost=4 Port=0x8002 | BPDU 8 of 18 |
| #1380 | 17.745 | f4:4d:5c:98:63:19 | STP | RST BPDU Root=32768/08:7b:12:e5:ef:07 Cost=4 Port=0x8002 | BPDU 9 of 18 |
| #1524 | 19.745 | f4:4d:5c:98:63:19 | STP | RST BPDU Root=32768/08:7b:12:e5:ef:07 Cost=4 Port=0x8002 | BPDU 10 of 18 |
| #1660 | 21.745 | f4:4d:5c:98:63:19 | STP | RST BPDU Root=32768/08:7b:12:e5:ef:07 Cost=4 Port=0x8002 | BPDU 11 of 18 |
| #2000 | 23.745 | f4:4d:5c:98:63:19 | STP | RST BPDU Root=32768/08:7b:12:e5:ef:07 Cost=4 Port=0x8002 | BPDU 12 of 18 |
| #2016 | 24.045 | f4:4d:5c:98:63:19 | LLDP | SysN=GS1915-8EP SysD=V4.70(ACAQ.6)¦12/19/2025 MgmtIP=192.168.1.1 | LLDP frame 1 of 9 |
| #2133 | 25.745 | f4:4d:5c:98:63:19 | STP | RST BPDU Root=32768/08:7b:12:e5:ef:07 Cost=4 Port=0x8002 | BPDU 13 of 18 |
| #2134 | 25.931 | f4:4d:5c:98:63:17 | ARP | Who has 192.168.1.254? Tell 192.168.1.1 [broadcast] | ARP request — no reply received |
| #2267 | 27.744 | f4:4d:5c:98:63:19 | STP | RST BPDU Root=32768/08:7b:12:e5:ef:07 Cost=4 Port=0x8002 | BPDU 14 of 18 |
| #2428 | 29.744 | f4:4d:5c:98:63:19 | STP | RST BPDU Root=32768/08:7b:12:e5:ef:07 Cost=4 Port=0x8002 | BPDU 15 of 18 |
| #2659 | 31.744 | f4:4d:5c:98:63:19 | STP | RST BPDU Root=32768/08:7b:12:e5:ef:07 Cost=4 Port=0x8002 | BPDU 16 of 18 |
| #2807 | 33.744 | f4:4d:5c:98:63:19 | STP | RST BPDU Root=32768/08:7b:12:e5:ef:07 Cost=4 Port=0x8002 | BPDU 17 of 18 |
| #2982 | 35.743 | f4:4d:5c:98:63:19 | STP | RST BPDU Root=32768/08:7b:12:e5:ef:07 Cost=4 Port=0x8002 | LAST frame from switch — BPDU 18 of 18 |
| ■ NO FRAMES FROM ZYXEL SWITCH BETWEEN t=35.743s AND t=102.423s ■ Gap = 66.68 seconds ■ |  |  |  |  |  |
| #3109 | 102.423 | f4:4d:5c:98:63:19 | LLDP | SysN=GS1915 SysD=V4.70(ACAQ.6)¦12/19/2025 MgmtIP=192.168.1.1 | FIRST frame after gap — SysName now 'GS1915' |
| #3149 | 103.623 | f4:4d:5c:98:63:19 | LLDP | SysN=GS1915 SysD=V4.70(ACAQ.6)¦12/19/2025 MgmtIP=192.168.1.1 | LLDP frame 3 of 9 |
| #3176 | 104.823 | f4:4d:5c:98:63:19 | LLDP | SysN=GS1915 SysD=V4.70(ACAQ.6)¦12/19/2025 MgmtIP=192.168.1.1 | LLDP frame 4 of 9 |
| #3233 | 106.346 | f4:4d:5c:98:63:17 | ARP | Who has 192.168.1.254? Tell 192.168.1.1 [broadcast] | ARP request — no reply received |
| #3811 | 130.073 | f4:4d:5c:98:63:19 | LLDP | SysN=GS1915 SysD=V4.70(ACAQ.6)¦12/19/2025 MgmtIP=192.168.1.1 | LLDP frame 5 of 9 |
| #4290 | 160.070 | f4:4d:5c:98:63:19 | LLDP | SysN=GS1915 SysD=V4.70(ACAQ.6)¦12/19/2025 MgmtIP=192.168.1.1 | LLDP frame 6 of 9 |
| #4465 | 190.066 | f4:4d:5c:98:63:19 | LLDP | SysN=GS1915 SysD=V4.70(ACAQ.6)¦12/19/2025 MgmtIP=192.168.1.1 | LLDP frame 7 of 9 |
| #4662 | 220.064 | f4:4d:5c:98:63:19 | LLDP | SysN=GS1915 SysD=V4.70(ACAQ.6)¦12/19/2025 MgmtIP=192.168.1.1 | LLDP frame 8 of 9 |
| #4900 | 250.060 | f4:4d:5c:98:63:19 | LLDP | SysN=GS1915 SysD=V4.70(ACAQ.6)¦12/19/2025 MgmtIP=192.168.1.1 | LLDP frame 9 of 9 — end of capture |

Total frames from Zyxel switch in 264.3 seconds:  29  (18 STP BPDUs, 9 LLDP, 2 ARP)

Total IP-layer frames from Zyxel switch:  0

Total DHCP frames from Zyxel switch:  0  (no Discover, Request, or Inform observed)

# 3.  Detailed Observations

## 3.1  LLDP Frames — TLV Content

Nine LLDP frames were transmitted by MAC f4:4d:5c:98:63:19. The System Name TLV value differs between the frame sent before the gap and all frames sent after the gap.

| Frame | Time (s) | System Name | System Description | Mgmt IP (IPv4) | Interval to prev LLDP |
| --- | --- | --- | --- | --- | --- |
| #2016 | 24.045 | GS1915-8EP | V4.70(ACAQ.6) ¦ 12/19/2025 | 192.168.1.1 | First LLDP in capture |
| #3109 | 102.423 | GS1915 | V4.70(ACAQ.6) ¦ 12/19/2025 | 192.168.1.1 | 78.378 s after #2016 |
| #3149 | 103.623 | GS1915 | V4.70(ACAQ.6) ¦ 12/19/2025 | 192.168.1.1 | 1.200 s |
| #3176 | 104.823 | GS1915 | V4.70(ACAQ.6) ¦ 12/19/2025 | 192.168.1.1 | 1.200 s |
| #3811 | 130.073 | GS1915 | V4.70(ACAQ.6) ¦ 12/19/2025 | 192.168.1.1 | 25.250 s |
| #4290 | 160.070 | GS1915 | V4.70(ACAQ.6) ¦ 12/19/2025 | 192.168.1.1 | 29.997 s |
| #4465 | 190.066 | GS1915 | V4.70(ACAQ.6) ¦ 12/19/2025 | 192.168.1.1 | 29.996 s |
| #4662 | 220.064 | GS1915 | V4.70(ACAQ.6) ¦ 12/19/2025 | 192.168.1.1 | 29.998 s |
| #4900 | 250.060 | GS1915 | V4.70(ACAQ.6) ¦ 12/19/2025 | 192.168.1.1 | 29.996 s |

Observation: Frame #2016 (pre-gap) carries System Name TLV value GS1915-8EP. All nine LLDP frames transmitted after the gap carry System Name TLV value GS1915. The System Description TLV (V4.70(ACAQ.6) | 12/19/2025) and the Management Address IPv4 TLV (192.168.1.1) are identical in all nine frames.

## 3.2  STP BPDU Frames

Eighteen STP Rapid Spanning Tree (RST) BPDUs were transmitted by MAC f4:4d:5c:98:63:19. All carried identical content. No STP BPDUs were observed after the gap.

STP BPDU field values (all 18 frames identical):

Protocol:         IEEE 802.1w Rapid Spanning Tree

Root Bridge:      Priority 32768  /  MAC 08:7b:12:e5:ef:07  (Sagemcom router)

Root Path Cost:   4

Bridge ID:        Priority 32768  /  MAC f4:4d:5c:98:63:17  (Zyxel switch)

Port Identifier:  0x8002

Frame timestamps:

#196  t=1.744s     #356  t=3.745s     #497  t=5.745s     #644  t=7.745s

#772  t=9.744s     #961  t=11.747s     #1066 t=13.747s     #1215 t=15.746s

#1380 t=17.745s     #1524 t=19.746s     #1660 t=21.745s     #2000 t=23.746s

#2133 t=25.745s     #2267 t=27.744s     #2428 t=29.745s     #2659 t=31.745s

#2807 t=33.745s     #2982 t=35.744s  <-- LAST STP BPDU IN CAPTURE

Observation: BPDUs are transmitted at approximately 2-second intervals from t=1.74s to t=35.74s (18 frames over 33.999 seconds). No STP BPDU frames are present from any Zyxel source MAC after t=35.744s for the remainder of the 264.3-second capture.

## 3.3  ARP Frames — Full Exchange Log

Two ARP request frames were transmitted by the Zyxel switch management MAC (f4:4d:5c:98:63:17). The table below shows all ARP activity in the capture involving the switch IP address 192.168.1.1 or switch MAC, showing both the request and whether a reply was observed.

| Frame | Time (s) | Sender | Target IP Query | Reply Observed? |
| --- | --- | --- | --- | --- |
| #1959 | 23.249 | 08:7b:12:e5:ef:07 (192.168.1.254) | Who has 192.168.1.1? | NO — no ARP reply from 192.168.1.1 in capture |
| #2134 | 25.931 | f4:4d:5c:98:63:17 (192.168.1.1) | Who has 192.168.1.254? | NO — no ARP reply from 192.168.1.254 in capture |
| #3232 | 106.343 | 08:7b:12:e5:ef:07 (192.168.1.254) | Who has 192.168.1.1? | NO — no ARP reply from 192.168.1.1 in capture |
| #3233 | 106.346 | f4:4d:5c:98:63:17 (192.168.1.1) | Who has 192.168.1.254? | NO — no ARP reply from 192.168.1.254 in capture |

Observation: The router (192.168.1.254) sends an ARP request for 192.168.1.1 at t=23.249s. Two seconds later at t=25.931s the switch sends an ARP request for 192.168.1.254. No ARP reply is transmitted by either device in response to these requests. This sequence repeats after the gap at t=106.343s and t=106.346s with the same result. No ARP reply frames from either 192.168.1.1 or 192.168.1.254 are present in the capture in response to queries for those addresses.

Note: The router does successfully respond to ARP requests for 192.168.1.2 (the HP laptop) in frames #2809, #3195, #4250, #4514, and #4775. These exchanges complete normally.

## 3.4  DHCP Activity

Four DHCP frames are present in the capture. All four originate from the HP laptop (e0:70:ea:52:a7:8a). Zero DHCP frames originate from either Zyxel switch MAC address.

| Frame | Time (s) | Source MAC | DHCP Type | Key Fields |
| --- | --- | --- | --- | --- |
| #3089 | 101.250 | e0:70:ea:52:a7:8a | Request (type 3) | Client ID: e0:70:ea:52:a7:8a Requested IP: 192.168.1.2 Hostname: Fieldlaptop2 |
| #3146 | 103.257 | e0:70:ea:52:a7:8a | Request (type 3) | Client ID: e0:70:ea:52:a7:8a Requested IP: 192.168.1.2 Hostname: Fieldlaptop2 |
| #3272 | 107.219 | e0:70:ea:52:a7:8a | Request (type 3) | Client ID: e0:70:ea:52:a7:8a Requested IP: 192.168.1.2 Hostname: Fieldlaptop2 |
| #3273 | 107.229 | 08:7b:12:e5:ef:07 | ACK (type 5) | Your IP: 192.168.1.2 Server: 192.168.1.254 Subnet: 255.255.255.0 Router: 192.168.1.254 |

Observation: No DHCP Discover (type 1), DHCP Request (type 3), DHCP Inform (type 8), or any other DHCP message type is transmitted by MAC f4:4d:5c:98:63:17 or f4:4d:5c:98:63:19 at any point in the capture.

## 3.5  Traffic Gap — t=35.744s to t=102.423s

The last frame transmitted by any Zyxel switch MAC is STP BPDU frame #2982 at t=35.744s. The next frame from any Zyxel MAC is LLDP frame #3109 at t=102.423s. The gap between these two frames is 66.679 seconds.

Last frame from Zyxel:   #2982  t=35.744s   STP BPDU (f4:4d:5c:98:63:19)

First frame after gap:   #3109  t=102.423s  LLDP     (f4:4d:5c:98:63:19)

Gap duration:  66.679 seconds

Frames from Zyxel in gap: 0

Observation: No frames of any protocol type are transmitted from f4:4d:5c:98:63:17 or f4:4d:5c:98:63:19 during this 66.679-second window. The next Zyxel frame (#3109) is an LLDP frame carrying System Name GS1915 — differing from the System Name GS1915-8EP carried in the only LLDP frame (#2016) transmitted before the gap.

## 3.6  IP Traffic — Zyxel Switch

A search of all 5,022 frames in the capture for any frame with ip.src matching 192.168.1.1 (the address advertised in LLDP and ARP) returns zero results. The switch transmits no IPv4 frames at any point during the 264.3-second capture.

tshark filter applied:  ip.src == 192.168.1.1

Result:                 0 frames matched

tshark filter applied:  eth.src == f4:4d:5c:98:63:17 or eth.src == f4:4d:5c:98:63:19

Protocol breakdown of 29 matched frames:

STP:   18 frames

LLDP:   9 frames

ARP:    2 frames  (Layer 2 only — no IP layer present in ARP)

TCP:    0 frames

UDP:    0 frames

ICMP:   0 frames

# 4.  Capture Statistics

| Capture duration | 264.3 seconds |
| --- | --- |
| Total frames | 5,022 |
| Total bytes | 2,989,942 |
| Capture start (UTC) | Apr 10, 2026 08:03:07.156 |
| Capture end (UTC) | Apr 10, 2026 08:07:31.432 |
| Protocol split (all frames) | TCP: 87.1% UDP: 11.6% STP: 0.36% LLDP: 0.18% ARP: 0.47% |
| Frames from Zyxel MACs | 29 of 5,022 (0.58%) |
| Frames with ip.src=192.168.1.1 | 0 |
| DHCP frames from Zyxel MACs | 0 |
| TCP frames from Zyxel MACs | 0 |

# 5.  Raw Frame Data — Key Frames

Verbatim tshark field output for frames of interest.

Frame #2016  —  LLDP (pre-gap, System Name = GS1915-8EP)

Source:      f4:4d:5c:98:63:19  ->  01:80:c2:00:00:0e  (LLDP Multicast)

Time:        t=24.045s

Chassis ID:  MAC  f4:4d:5c:98:63:17

Port ID:     Locally assigned  '2'

TTL:         120 seconds

System Name: GS1915-8EP

System Desc: V4.70(ACAQ.6) | 12/19/2025

Mgmt Addr:   192.168.1.1  (IPv4)

Mgmt Addr:   f4:4d:5c:98:63:17  (MAC)

Frame #3109  —  LLDP (post-gap, System Name = GS1915)

Source:      f4:4d:5c:98:63:19  ->  01:80:c2:00:00:0e  (LLDP Multicast)

Time:        t=102.423s  (66.679s after last Zyxel frame)

Chassis ID:  MAC  f4:4d:5c:98:63:17

Port ID:     Locally assigned  '2'

TTL:         120 seconds

System Name: GS1915

System Desc: V4.70(ACAQ.6) | 12/19/2025

Mgmt Addr:   192.168.1.1  (IPv4)

Mgmt Addr:   f4:4d:5c:98:63:17  (MAC)

Frame #2134  —  ARP Request (switch, no reply)

Source:       f4:4d:5c:98:63:17  ->  ff:ff:ff:ff:ff:ff  (Broadcast)

Time:         t=25.931s

Opcode:       Request (1)

Sender MAC:   f4:4d:5c:98:63:17

Sender IP:    192.168.1.1

Target MAC:   00:00:00:00:00:00

Target IP:    192.168.1.254

Reply in capture: NONE

Frame #3233  —  ARP Request (switch post-gap, no reply)

Source:       f4:4d:5c:98:63:17  ->  ff:ff:ff:ff:ff:ff  (Broadcast)

Time:         t=106.346s

Opcode:       Request (1)

Sender MAC:   f4:4d:5c:98:63:17

Sender IP:    192.168.1.1

Target MAC:   00:00:00:00:00:00

Target IP:    192.168.1.254

Reply in capture: NONE

Frame #1959  —  ARP Request (router querying for switch, no reply)

Source:       08:7b:12:e5:ef:07  ->  ff:ff:ff:ff:ff:ff  (Broadcast)

Time:         t=23.249s

Opcode:       Request (1)

Sender MAC:   08:7b:12:e5:ef:07

Sender IP:    192.168.1.254

Target IP:    192.168.1.1

Reply in capture: NONE

End of Report